Nama		: Royyan Candrawinata

Kelas		: 1B-TT

No.Absen	: 17

Project 2 MQTT

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <SimpleDHT.h>

#include <WiFi.h>

#include <PubSubClient.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET    -1

#define pinDHT 34

#define pinLEDHijau 13

#define pinLEDMerah 12

// Kredensial WiFi

const char* ssid = "Polinema Hotspot";

const char* password = " ";

// Kredensial MQTT Broker

const char* mqtt_server = "broker.emqx.io";

const int mqtt_port = 1883;

const char* mqtt_topic_temp = "esp32/temperature";

const char* mqtt_topic_hum = "esp32/humidity";

WiFiClient espClient;

PubSubClient client(espClient);

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

SimpleDHT11 dht11(pinDHT);

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

}

void reconnect() {

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

if (client.connect("ESP32Client")) {

Serial.println("connected");

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void setup() {

Serial.begin(115200);

// Inisialisasi OLED

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("SSD1306 allocation failed"));

for(;;);

}

display.display();

delay(2000);

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0,0);

// Inisialisasi LED

pinMode(pinLEDHijau, OUTPUT);

pinMode(pinLEDMerah, OUTPUT);

// Setup WiFi dan MQTT

setup_wifi();

client.setServer(mqtt_server, mqtt_port);

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

byte temperature = 0;

byte humidity = 0;

int err = SimpleDHTErrSuccess;

// Membaca data dari DHT11

if ((err = dht11.read(&temperature, &humidity, NULL)) !=

SimpleDHTErrSuccess) {

Serial.print("Read DHT11 failed, err="); Serial.println(err);

delay(1000);

return;

}

// Menampilkan data suhu dan kelembaban pada OLED

display.clearDisplay();

display.setCursor(0,0);

display.print("Temperature: ");

display.print((int)temperature);

display.println(" C");

display.print("Humidity: ");

display.print((int)humidity);

display.println(" H");

display.display();

// Kontrol LED berdasarkan kelembaban

if (humidity < 69) {

digitalWrite(pinLEDHijau, HIGH);

digitalWrite(pinLEDMerah, LOW);

} else {

digitalWrite(pinLEDHijau, LOW);

digitalWrite(pinLEDMerah, HIGH);

}

// Publish data suhu dan kelembaban ke MQTT

char tempString[8];

snprintf(tempString, sizeof(tempString), "%d", (int)temperature);

client.publish(mqtt_topic_temp, tempString);

Serial.print("Published Temperature: ");

Serial.println(tempString);

char humString[8];

snprintf(humString, sizeof(humString), "%d", (int)humidity);

client.publish(mqtt_topic_hum, humString);

Serial.print("Published Humidity: ");

Serial.println(humString);

delay(2000); // Jeda 2 detik sebelum membaca dan mempublish data lagi

}

FOTO PROJECT